# CHII: Curb Cut Measurements

David Ellsworth: So, we want to make sure that the slope of the curb cut is OK, less than 8 1/2 percent or 4.7°. Is it free of barriers, hazards, breaks in the surface, and is it in good condition? So, there are curb cuts here. They appear to be in very good condition. Often times these are made out of plastic or some other kind of composite. So, you're going to want to make sure that they are not worn out, that they're not peeling, that they're not coming up, and that there's not, you know, big chunks missing because that could make it really difficult to walk over, or between, or about. So, good working condition.

We'll come over here towards our park and we'll go ahead and measure to make sure that the slope is the right amount. So, you'll want to try to position this very level if you have a level that might be ideal. Otherwise, you can usually situate it in between these truncated domes. We can see that this is less than 8.3%, so it's not too steep. People can easily get around it.